Performance Qualification Template

- Introduction

  - Objectives

The objective of Performance Qualification (PQ) testing is to demonstrate that the solution operates under normal expected conditions with acceptable and expected output results.

  - Scope

PQ testing involves performing basic and interactive tasks that involve specifications that are defined in the Nymi Design/Configuration Specifications document.

  - Assumptions

The tests in this document assume that the following infrastructure is in place before testing begins:

- NES is deployed in the environment.

- The Nymi Band application has been installed on at least one machine that has network connectivity with NES.

- An MES is installed and configured on at least one machine that has network connectivity with NES.

- A reliable network connection exists between each component in the environment.

- The tester has access to Nymi Connected Worker Platform documentation.

  - Exclusions

- Acronyms and References

  - Acronyms and Definitions

| Acronyms and Definitions |  |
| --- | --- |
| AC | Acceptance Criteria |
| CFR | Code of Federal (US) Regulations |
| cGxP | Abbreviation which includes current Good Manufacturing, Clinical and Laboratory Practices |
| CS | Configuration Specification |
| CSV | Computer Systems Validation |
| DS | Design Specification |
| FS | Functional Specification |
| GUI | Graphical User Interface |
| IQ | Installation Qualification |
| LAN | Local Area Network |
| OQ | Operational Qualification |
| PQ | Performance Qualification |
| SOP | Standard Operating Procedure |
| URS | User Requirements Specification |

  - References

| References |  |
| --- | --- |
| 21 CFR | Part 11, Part 210 |
| GAMP5 | Guide for Validation of Automated Systems |
| URS | URS for Biometric MES System |
| FS | NYMI FS-001 |
| DS/CS | NYMI CS-001 |

- Responsibilities

  - System Owner

To provide all required documentation, create the performance qualification testing document, and to provide a functional test environment.

  - System Validation Tester

To perform the qualification testing and record the results in this document.  Documenting any issues that are encountered.

- System Description

NES and AD servers reside in the same domain as do the user terminals. The configuration falls into GAMP5 Category 3.

- Test Procedure

  - General

| System Name: | Nes-and-evidian.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | NES installation tests |  |  |  |
| Purpose | Validate that the NES is functional in the IT Infrastructure |  |  |  |
| Test Reference: |  | PQ-001 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| URS tested: URS-001 Functional specification: FS-CFG-01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Locate the Nymi Band Application and follow the steps on the interface to enroll your Nymi Band. | Enrollment is successful. | Enrollment succeeds. | Pass |
| 2 | Log in to the NES Admin Console with an administrator account. | Log is successful. | Log succeeds. | Pass |
| 3 | Navigate to the About page, and review the system Diagnostics | System Diagnostics all Pass | All system diagnostics pass | Pass |

| System Name: | Nes-and-evidian.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing the Nymi Band battery |  |  |  |
| Purpose | Test the Nymi Band to confirm a 3-day battery life |  |  |  |
| Test Reference: |  | PQ-002 | Test Run #: | 02 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| URS tested: URS-007 Functional specifications: FS-BAT-001 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | DAY1: Charge your Nymi Band for at least 2 hours. Record the amount of time that the Nymi Band was charging for in the actual results for this step. | Tester records the amount of time the Nymi Band is charged. | Nymi Band left on charge for 2 hours. | Pass |
| 2 | Wake Nymi Band while it is still attached to charger. | The Nymi Band display clearly displays that the band is fully charged. | A full battery icon appears on the Nymi Band screen. | Pass |
| 3 | Log into the Nymi Band Application and follow the instructions to enroll your Nymi Band. | Enrollment is successful. | Enrollment succeeds. | Pass |
| 4 | Record the date and time that you enrolled your Nymi Band for your first authentication event/first tap. | Time is recorded. | Nymi Band enrolled at 8:34 | Pass |
| 5 | Throughout the 10-hour shift, open the NFC tap application and perform 300 authentication events by tapping the Nymi Band against an NFC reader. | The battery lasts throughout the 10-hour shift. The Nymi Band does not falsely deauthenticate. | Nymi Band remained authenticated the entire 10 hours shift, Nymi Band was tapped 300 times over the course of the shift. | Pass |
| 6 | Take off the Nymi Band and put it on a table. | The Nymi Band deauthenticates within 3 seconds of removal. | Nymi Band deauthenticated in 1 second. | Pass |
| 7 | Attempt to authenticate the Nymi Band while it is on the table. Leave for the day. | The Nymi Band does not authenticate. | Nymi Band authentication failed | Pass |
|  | DAY 2: |  |  |  |
| 8 | Record the time of your first authentication event/first tap. | Time is recorded. | First tap occurred at 9 am. | Pass |
| 9 | Throughout the 10-hour shift, open the NFC tap application and perform 300 authentication events by tapping the Nymi Band against an NFC reader. | The battery lasts throughout the 10-hour shift. The Nymi Band does not falsely deauthenticate. | Nymi Band remained authenticated the entire 10 hours shift, Nymi Band was tapped 300 times over the course of the shift. | Pass |
| 10 | Take off the Nymi Band, leave it unauthenticated. and untouched, and then leave for the day. | The Nymi Band deauthenticates within 3 seconds of removal. | Nymi Band deauthenticated in 1.2 seconds. | Pass |
|  | DAY 3: |  |  |  |
| 11 | Record the time of your first authentication event/first tap. | Time is recorded. | First authentication occurred at 8:56 am | Pass |
| 12 | Throughout the 10-hour shift, open the NFC tap application and perform 300 authentication events by tapping the Nymi Band against an NFC reader. | The battery lasts throughout the 10-hour shift. The Nymi Band does not falsely deauthenticate. | Nymi Band remained authenticated the entire 10 hours shift, Nymi Band was tapped 300 times over the course of the shift. | Pass |
| 13 | Take off the Nymi Band, leave it unauthenticated and untouched, and then leave for the day. | The Nymi Band deauthenticates within 3 seconds of removal. | Nymi Band deauthenticated in 2 seconds. | Pass |
| 14 | Leave the Nymi Band off charge and observe the battery level. | The Nymi Band screen displays the battery level. | Nymi Band displays a one bar battery level. | Pass |
|  | Days 4-8: |  |  |  |
| 15 | When the battery dies, wait until the 7th day after the battery was charged to put the Nymi Band no charge. | The Nymi Band displays the charging symbol. | The charging symbol appeared on the Nymi Band when put on charge. | Pass |
| 16 | When the Nymi Band is fully charged, put the Nymi Band on your wrist. and authenticate with your fingerprint. | The Nymi Band indicates that the Nymi Band is fully charge and prompts the user to authenticate. | The Nymi Band displayed a full battery charge. Authentication succeeded on the first attempt and the Nymi Band displays 'READY'. | Pass |