# Operational Qualification Template

  - Objective

The objective of Operational Qualification (OQ) testing is to demonstrate that the solution is operable under normal expected conditions with acceptable and expected output results.

  - Scope

OQ testing involves using a Nymi solution to perform day-to-day activities that are defined in the Nymi Design/Configuration Specifications document.

  - Assumptions

    - A reliable network connection exists between each component in the environment.

    - The tester has access to Nymi Connected Worker Platform documentation.

  - Exclusions

n/a

- Acronyms and References

  - Acronyms and Definitions

| Acronyms and Definitions |  |
| --- | --- |
| AC | Acceptance Criteria |
| CFR | Code of Federal (US) Regulations |
| cGxP | Abbreviation which includes current Good Manufacturing, Clinical and Laboratory Practices |
| CS | Configuration Specification |
| CSV | Computer Systems Validation |
| DS | Design Specification |
| FS | Functional Specification |
| GUI | Graphical User Interface |
| IQ | Installation Qualification |
| LAN | Local Area Network |
| OQ | Operational Qualification |
| PQ | Performance Qualification |
| SOP | Standard Operating Procedure |
| URS | User Requirements Specification |

  - References

| References |  |
| --- | --- |
| 21 CFR | Part 11, Part 210 |
| GAMP5 | Guide for Validation of Automated Systems |
| URS | URS for Biometric MES System |
| FS | NYMI FS-001 |
| DS/CS | NYMI CS-001 |

- Responsibilities

  - System Owner

To provide all required documentation, create the performance qualification testing document, and to provide a functional test environment.

  - System Validation Tester

To perform the qualification testing and record the results in this document.  Documenting any issues that are encountered.

- System Description

Unless otherwise noted, NES and AD servers reside in the same domain as do the user terminals.

Each User terminal has one attached Nymi-supported NFC reader and one attached Bluegiga BLE adapter.

The configuration falls into GAMP5 Category 3.

- Test Procedure

  - General

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing NES operations |  |  |  |
| Purpose | Validate that the NES is functional in the IT Infrastructure and that a user can enroll in the Nymi Band. |  |  |  |
| Test Reference: |  | OQ-001 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-001, URS-002 Functional Specification: FS-CFG-001, FS-CFG-010 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Connect to the NES Administration console. | A secure NES Console webpage appears. | Enter the NES Console URL. NES Console appears successfully | Pass |
| 2 | Log into the NES Administration Console with an administrator account. | Log in succeeds. | User successfully login to the NES Console | Pass |
| 3 | Navigate to the About page and click View Full System Diagnostics. | The System Diagnostics page does not report errors. | System Diagnostic ran successfully without any errors. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing operations in a multi-domain environment |  |  |  |
| Purpose | Validate that the NES is functional in a multidomain environment, users in different domains can enroll the Nymi Band to the same NES server, and NES administrators in different domains can manage NES. |  |  |  |
| Test Reference: |  | OQ-002 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-003, URS-024, URS-039 Functional Specification: FS-CFG-03, FS-CFG-04, FS-APP-002 Configuration details: NES is installed in domain A. Users are in domain A, B, C. Domain A is a root domain and B is A's sub-domain. Domain C is the root domain of another forest. There is a two-way trust from A to C. |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Use a computer and user account on Domain A to enroll a Nymi Band to NES using Nymi Band Application. | Enrollment succeeds. | Enrollment succeeded. | Pass |
| 2 | Use a computer and user account on Domain B to enroll a Nymi Band to NES using Nymi Band Application. | Enrollment succeeds. | Enrollment succeeded. | Pass |
| 3 | Use a computer and user account on Domain C to enroll a Nymi Band to NES using Nymi Band Application. | Enrollment succeeds. | Enrollment succeeded. | Pass |
| 4 | Connect to the NES Administration console. | A secure NES Console webpage appears. | Page appeared. | Pass |
| 5 | Log into the NES Administration Console with an administrator account in Domain A. | Log in succeeds. | Login succeeded. | Pass |
| 6 | Search for Users in all three domains. | Each search result, returns a valid user in each domain. | All search results displayed a valid user. | Pass |
| 7 | Open Policies page to view policies. | Policies pages opens successfully. | Page opened successfully. | Pass |
| 8 | Open the default policy and edit and save the changes. | Policy save succeeds. | Policy saved successfully. | Pass |

| System Name: | Nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing that the solution supports multiple authentication methods |  |  |  |
| Purpose | Validate that Connected Worker Platform supports authentication by AD credentials and/or fingerprint verification. |  |  |  |
| Test Reference: |  | OQ-003 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-030 Functional Specification: FS-NB-015, FS-APP-001 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Instruct a user to use the Nymi Band Application to enroll a Nymi Band. | Enrollment succeeds. | Nymi Band displays a checkmark(successful authentication) and battery sign. | Pass |
| 2 | Instruct the user to remove the Nymi Band. | Nymi Band de-authentication succeeds. | Nymi Band vibrates and displays the user label(name) and battery sign. | Pass |
| 3 | Instruct the user to place the Nymi Band back on the same wrist that was used during the enrollment process. | Nymi Band prompts the user to authenticate. | Nymi Band displays a fingerprint sign prompting the user to authenticate. | Pass |
| 4 | Instruct the user to place the finger that they used to enroll the Nymi Band on the fingerprint sensor for the amount of time indicated on the Nymi Band screen. | Nymi Band authentication succeeds. | Nymi Band displays a checkmark and battery sign. | Pass |
| 5 | Instruct the user to remove the Nymi Band. | Nymi Band de authenticates. | Nymi Band vibrates and displays user label and battery sign on. | Pass |
| 6 | Instruct the user to log in to the Nymi Band application using their corporate credentials. | Login is successful. | Log in succeeds. | Pass |
| 7 | Instruct the user to click the Authenticate button. | Nymi Band authentication succeeds. | Nymi Band is authenticated and the band displays a checkmark and battery sign. | Pass |

| System Name: | Nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing solution interactions with Active Directory. |  |  |  |
| Purpose | Validate that Nymi Connected Worker Platform solution ensures that the Nymi Band user is valid in Active Directory. Usernames and passwords are not stored by NES. |  |  |  |
| Test Reference: |  | OQ-004 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-013 Functional Specification: FS-NB-016 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Instruct a user to use the Nymi Band Application to enroll a Nymi Band. | Enrollment succeeds. | Nymi Band displays a checkmark (successful enrollment) and battery sign. | Pass |
| 2 | Instruct the IT admin to log in to a DC and disable the AD account for the user that is associated with the Nymi Band. | The account is disabled. | Disabling the user account succeeds. | Pass |
| 3 | Instruct the user to remove the Nymi Band and put it back on. | Nymi Band is deauthenticated. | Nymi Band displays the user label and battery sign. | Pass |
| 4 | Instruct the IT admin to log into a workstation. | Login is successful. | Log in succeeds. | Pass |
| 5 | Instruct the user to log into the Nymi Band application on the workstation. | Log in fails with an error. | Log in fails. | Pass |
| 6 | Instruct the IT admin to re-enable the user’s AD account. | Account is enabled. | Enabling the user account succeeds. | Pass |
| 7 | Instruct the user to log into the Nymi Band Application. | Login is successful in the Nymi Band Application. | Nymi Band Application displays the Authenticate button. | Pass |
| 8 | Instruct the user to click the Authenticate button. | Authentication succeeds. | Nymi Band is authenticated and displays a checkmark and battery sign. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing that the Nymi solution has an interface for enrollment. |  |  |  |
| Purpose | Validate that Nymi Band application provides users with an interface to associate Nymi Bands with users. |  |  |  |
| Test Reference: |  | OQ-005 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| URS tested: URS-026, URS-030, URS-039 Functional specification: FS-NB-015, FS-APP-001, FS-PHY-007 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Locate the Nymi Band Application and follow the steps on the interface to enroll your Nymi Band. | Enrollment is successful. | Nymi Band displays a check mark(successful enrollment) and battery sign. | Pass |
| 2 | Take out the Nymi Band from the user’s wrist. | Nymi Band de-authentication succeeds. | Nymi Band vibrates and displays the user label(name) and battery sign. | Pass |
| 3 | Put the Nymi Band on your wrist. | Nymi Band prompts the user to authenticate. | Nymi Band displays a fingerprint icon prompting the user to authenticate. | Pass |
| 4 | Log in to the Nymi Band Application using the same credentials that you used when you enrolled your Nymi Band. | Log in succeeds. | Nymi Band Application displays the Authenticate button. | Pass |
| 5 | Authenticate the Nymi Band by pressing the “Authenticate” Button in the Nymi Band Application. | Authentication is successful. | Nymi Band authentication succeeds. | Pass |
| 6 | Take out the Nymi Band from the user’s wrist. | Nymi Band de-authentication succeeds. | Nymi Band vibrates and displays the user label with battery sign. | Pass |
| 7 | Put the Nymi Band on your wrist. | Nymi Band prompts the user to authenticate. | Nymi Band displays a fingerprint icon prompting the user to authenticate . | Pass |
| 8 | Put the finger that you used to enroll the Nymi Band on the fingerprint reader and follow the instructions to authenticate the Nymi Band. | Authentication is successful. | Nymi Band is authenticated with a successful fingerprint. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Test removal of biometric information from the Nymi Band. |  |  |  |
| Purpose | Validate that Nymi Connected Worker Platform provides users with the ability to remove biometric information Nymi Band. |  |  |  |
| Test Reference: |  | OQ-006 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: USR-012 Functional specification: FS-NB-012 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Instruct a user to use the Nymi Band Application to enroll a Nymi Band. | Enrollment is successful. | Nymi Band displays a checkmark and battery sign. | Pass |
| 2 | Instruct the user to remove the Nymi Band. | Nymi Band de-authentication succeeds. | Nymi Band vibrates and the band displays user label and battery sign . | Pass |
| 3 | Instruct the user to place the Nymi Band back on the same wrist that was used during the enrollment process. | Nymi Band prompts the user to authenticate. | Nymi Band displays a fingerprint icon prompting the user to authenticate. | Pass |
| 4 | Instruct the user to place the finger that they used to enroll the Nymi Band on the fingerprint sensor for the amount of time indicated on the Nymi Band screen. | Nymi Band authentication succeeds. | Nymi Band is authenticated with a successful fingerprint. | Pass |
| 4 | Instruct the user to remove the Nymi Band. | Nymi Band de-authenticates. | Nymi Band vibrates and displays the user label and battery sign. | Pass |
| 5 | Instruct the user to plug the Nymi Band in to charge. | Nymi Band starts to charge. | Nymi Band screen displays the charging icon. | Pass |
| 7 | Instruct the user to hold the bottom button of the Nymi Band until the Deleting User Data appears. | Nymi Band data is successfully deleted. | Nymi Band screen displays “Deleting User Data”, boot sequence information and then displays No User and battery sign. | Pass |
| 8 | Instruct the user to put on the Nymi Band. | Nymi Band does not prompt the user to authenticate. | Nymi Band displays the setup code. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing firmware updates |  |  |  |
| Purpose | Validate that firmware updates can be performed concurrently on up to 5 Nymi Bands and without Nymi assistance. |  |  |  |
| Test Reference: |  | OQ-007 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-025, URS-009 Functional specification: FS-BAT-006, FS-BAT-005 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Obtain a USB charging hub that has at least 10 ports. | n/a | n/a | n/a |
| 2 | Plug 5 Nymi Bands and 5 Bluetooth adapters into the USB hub and ensure that the hub is plugged into a power supply. | Nymi Bands start to charge. | Nymi Bands display the charging symbol. | Pass |
| 3 | Place the hub within Bluetooth range of a computer that has the Nymi Band Application software installed and a Bluetooth adapter plugged into it. | n/a | n/a | n/a |
| 4 | From a command prompt on the computer, type the command to start the firmware updater script. | The firmware updater script starts, and the Nymi Band displays a message to indicate that the update is in progress. | The firmware updater displays a status window for the firmware updates and the Nymi Bands screens display “STANDBY”. | Pass |
| 5 | While the Nymi Band is displays "STANDBY", take the Nymi Band off charge. | Firmware update stops and Nymi Band does not display an error. | Nymi Band does not display "STANDBY" or report an error. | Pass |
| 6 | Leave the Nymi Band off charge for 5 mins. | The firmware update does not start. | Nymi Band does not display "STANDBY". | Pass |
| 7 | Put the Nymi Band back on charge. | The update of the Nymi Band firmware starts again. | Nymi Band displays "STANDBY". |  |
| 8 | Monitor the Nymi Band display and the firmware updater status window. | Nymi Band firmware updates complete and the administrator is provided with visual indications. | Nymi Bands reboot as each update completes and the status window provides information about the number of completed Nymi Bands. | Pass |
| 9 | In Windows Explorer, go to the Properties of the firmware updater script file. | The properties of the file provide signing information. | Signing information appears on the Digital Signatures tab. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing Nymi Band cleaning |  |  |  |
| Purpose | Validate that the Nymi Band can be sanitized with an alcohol wipe or spray. |  |  |  |
| Test Reference: |  | OQ-008 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-008 Functional specification: FS-ENV-003 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Using a wipe that has been soaked in 70% isopropanol, wipe all surfaces of the Nymi Band’s body including: -Front surface (display, fingerprint sensor) -Back surface (ECG electrode, sensor glass) -Sides (button surfaces, USB port) -exposed strap surfaces | Nymi Band is cleaned, and the body of the Nymi Band is undamaged by cleaning. | Nymi Band displays battery symbol and the body appears unchanged. | Pass |
| 2 | Wear the Nymi Band on your wrist and authenticate it using your fingerprint. | Nymi Band prompts the user to authenticate and then authentication is successful. | Nymi Band is authenticated following successful fingerprint. | Pass |
| 3 | Remove Nymi band off the wrist | Nymi Band de authenticates. | Nymi Band vibrates, deauthenticates and then displays user label and battery sign. | Pass |
| 4 | Repeat these 3 steps several times throughout the day. | Same results are expected. | All results remain the same. | Pass |
| 5 | Place the Nymi Band to charge. | Nymi Band starts to charge. | Nymi Band displays a battery icon that is full or has a lightning bolt. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing Nymi solution auditing |  |  |  |
| Purpose | Validate that the system maintains an audit log of Nymi Band user assignments |  |  |  |
| Test Reference: |  | OQ-009 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-027 Functional Specifications: FS-SAF-005 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Install an application that supports T-SQL queries, such as SSMS. | Installation completes without error. | Installation completes | Pass |
| 2 | Log in to the NES server with a user that has access to query the NES database. | Log in succeeds. | Login succeeds | Pass |
| 3 | Open SSMS and connect to the database. | Connection completes without error. | No errors | Pass |
| 4 | Query the database UserCore and NymiBand tables to gather information about the Nymi Band to user correlation. | Summary output appears that provides the username that is associated with each Nymi Band. | Summary provides sufficient details | Pass |

| System Name: | Nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Test the dissociation of a user from their Nymi Band. |  |  |  |
| Purpose | Validate that IT admin can dissociate a user from a Nymi Band. |  |  |  |
| Test Reference: |  | OQ-010 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-019 Functional specification: FS-APP-002 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Instruct a user to use the Nymi Band Application to enroll a Nymi Band. | Enrollment successful with the nymi band. | Nymi band displays a checkmark and battery sign. | Pass |
| 2 | IT admin logs into the NES Administrator Console. | Log in succeeds. | Login successful | Pass |
| 3 | IT admin searches for the username of the user. | Search results return the user | User is found | Pass |
| 4 | IT admin selects the user. | The User properties screen appears and the Nymi Band table displays information about the Nymi Band. | Properties populate | Pass |
| 4 | IT admin clicks Disconnect beside the Nymi Band entry in the Nymi Band table. | The Disconnect Nymi Band page appears. | Page appears | Pass |
| 5 | IT admin clicks Disconnect. | The Edit User page appears and the Nymi Band table does not contain an entry for the Nymi Band. | Page is accurate | Pass |
| 7 | Instruct the user log in to the Nymi Band Application. | Nymi Band Application prompts the user for the setup code. | Setup code displays on Nymi Bad | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Test the installation of an NEA on a Windows 10 thin client. |  |  |  |
| Purpose | Validate that an NEA can be installed on a Windows 10 thin client. |  |  |  |
| Test Reference: |  | OQ-011 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-021 Functional specification: FS-RDP-005 |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Follow the instructions in the Nymi Connected Worker Platform Deployment Guide to install the Nymi Agent application on the NES host. | Nymi Agent application installed successfully. | Installation succeeds. | Pass |
| 2 | Follow the instructions in the Nymi Connected Worker Platform Deployment Guide to install the Nymi Bluetooth Endpoint application on the Citrix or RDP client. | Installation completes successfully. | Installation succeeds. | Pass |
| 3 | Follow the instructions in the Nymi Connected Worker Platform Deployment Guide to create the nbe.toml file on the Citrix or RDP client. | nbe.toml file appears in the directory. | File creation succeeds. | Pass |
| 4 | Follow the instructions in the Nymi Connected Worker Platform Deployment Guide to install the NEA. | Installation completes successfully. | Installation succeeds. | Pass |
| 5 | Put a Nymi Band on your wrist and authenticate it. | Nymi Band prompts the user to authenticate. | Authentication succeeds. | Pass |
| 6 | Start the NEA. | NEA prompts you for your username and password. | The Login screen appears. | Pass |
| 7 | Tap the Nymi Band against the NFC reader. | The NEA login completes successfully. | Login succeeds. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing the Nymi solution with thin clients with NEAs and MES applications. |  |  |  |
| Purpose | Validate that an NEA can be installed on a Windows 10 thin client and the Nymi Band can be used to perform authentication tasks. |  |  |  |
| Test Reference: |  | OQ-012 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-004, URS-006, URS-022, URS-023, URS-014, URS-015, URS-017 Functional specification: FS-RDP-005, FS-MES-001, FS-MES-006, FS-NB-019, FS-MES-001 Configuration information: Tester is wearing a class A/C, C or D PPE suit. The NFC reader is covered with 3 cm of plexiglass. |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Following the steps provided by the MES vendor to install the MES application. | Installation completes successfully. | Installation succeeds. | Pass |
| 2 | Follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll a Nymi Band. | Enrollment completes successfully. | Enrollment succeeds. | Pass |
| 3 | Open the MES application. | The application prompts you to login. | Login screen appears. | Pass |
| 4 | Tap the authenticated Nymi Band against a connected NFC reader. | The application log in completes successfully. | Log in succeeds. | Pass |
| 5 | Perform an authentication task in the MES application. | The MES application prompts you for your username and password. | Username and Password prompts appear. | Pass |
| 6 | Tap the authenticated Nymi Band against a connected NFC reader. | The authentication task in the MES application completes successfully. | MES records the authentication event. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing the secure Nymi Band association with user during enrollment |  |  |  |
| Purpose | Validate that the Nymi Band Application (NBA) requires the user to provide consent when enrolling a Nymi Band. |  |  |  |
| Test Reference: |  | OQ-013 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-042 Functional specification: FS-ENR-009 Configuration information: User is wearing a charged, un-enrolled Nymi Band. |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Instruct the user to open the Nymi Band Application | Nymi Band applications opens. | Nymi Band applications opens. | Pass |
| 2 | Instruct the user to tap the Nymi Band to the dongle | Prompt the user to login to the Nymi band application. | Username and Password prompts appear. | Pass |
| 3 | Instruct the user to login with their Active Directory username and password | The user entered the username and password to login. | The username and password were entered, and login is successful. | Pass |
| 4 | Instruct the user tap the Nymi Band to the dongle to accept request consent. | The user accepts the request consent that user is assigned to this band. | The user accepts the consent for band and user association. | Pass |
| 5 | Instruct the user to perform the fingerprint capture process and authentication training. | The fingerprint print capture and authentication training are successful for this user. | The fingerprint print capture and authentication training are successful for this user. | Pass |
| 6 | The Nymi Band pushed the certs to the band and band is updated | Enrollment is completed successfully. | Enrollment is completed and the user disconnects. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing how to disable the haptic signals on the Nymi Band |  |  |  |
| Purpose | Validate the disabling of the haptic signals on a Nymi band. |  |  |  |
| Test Reference: |  | OQ-014 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-040 Functional specification: FS-ENV-005 Configuration information: User is wearing a charged, unenrolled Nymi Band |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Follow the Administration Guide and disable the haptic feedback on the Nymi band. | The haptic feedback is disabled in the policy. | The haptic feedback is disabled in the policy. | Pass |
| 2 | Follow the Administration Guide and enroll a Nymi Band. | Nymi Band is successfully enrolled without haptic feedback. | Nymi Band is successfully enrolled and does not vibrate upon completion of the enrollment. | Pass |
| 3 | Remove the Nymi band from user’s arm. | The Nymi band is removed from the user’s arm without haptic feedback. | The Nymi band is removed from the user’s arm and does not vibrate. | Pass |
| 4 | Place the Nymi Band to the user’s arm and authenticate | Nymi Band authenticated successfully without haptic feedback. | Nymi Band authenticated successfully without haptic feedback and does not vibrate. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing Nymi Band taps to perform e-signatures in a web-based MES from an iOS device |  |  |  |
| Purpose | Verify that users can use their Nymi Band to perform e-signatures in web application that they access from an iOS device. |  |  |  |
| Test Reference: |  | OQ-015 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-041 Functional specification: FS-WEB-010 Configuration information: Web application is configured to use the Nymi Application. |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Following the steps provided by the MES vendor to install the MES web application. | Installation completed successfully. | Installation succeeds. | Pass |
| 2 | Follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll a Nymi Band. | Enrollment is completed successfully. | Enrollment succeeds. | Pass |
| 3 | Launch the MES web application. | The web application is opened. | The web application is opened. | Pass |
| 4 | Tap the authenticated Nymi Band against the iOS device. | The e-signature is created successfully. | The e-signature is created successfully. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing Self- Service re-enrollment |  |  |  |
| Purpose | Verify that users can re-enroll their Nymi Band while maintaining organization level access control |  |  |  |
| Test Reference: |  | OQ-016 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 06-03-2024 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-039, URS-043 Functional specification: FS-APP-003, FS-APP-004 Configuration information: Self – Serve enrollment is configured and usable |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | As the Site – Admin or Supervisor, follow the instructions in the Nymi Connected Worker Platform Administration Guide activate the policy to enable self-service re-enrollment policy. | Self- Service re-enrollment is enabled in the policy. | Self- Service re-enrollment is enabled in the policy. | Pass |
| 2 | As the End User, follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll a Nymi Band. | Enrollment is completed successfully. | Enrollment succeeds. | Pass |
| 3 | Open the MES application. | The application prompts you to login. | Login screen appears. | Pass |
| 4 | Tap the authenticated Nymi Band against a connected NFC reader. | The application log in completes successfully. | Log in succeeds. | Pass |
| 5 | Perform an authentication task in the MES application. | The MES application prompts you for your username and password. | Username and Password prompts appear. | Pass |
| 6 | Tap the authenticated Nymi Band against a connected NFC reader. | The authentication task in the MES application completes successfully. | MES records the authentication event. | Pass |
| 7 | As the End User, follow the instructions in the Nymi Connected Worker Platform Administration Guide to delete the user on the Nymi Band. | User deletion is completed successfully on the Nymi Band. | User deletion is completed successfully on the Nymi Band. | Pass |
| 8 | As the End User, follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll a Nymi Band. | Enrollment is completed successfully. | Enrollment is completed successfully. | Pass |
| 9 | Open the MES application. | The application prompts you to login. | Login screen appears. | Pass |
| 10 | Tap the authenticated Nymi Band against a connected NFC reader. | The application log in completes successfully. | Log in succeeds. | Pass |
| 11 | Perform an authentication task in the MES application. | The MES application prompts you for your username and password. | Username and Password prompts appear. | Pass |
| 12 | Tap the authenticated Nymi Band against a connected NFC reader. | The authentication task in the MES application completes successfully. | MES records the authentication event. | Pass |
| 13 | As the End User, follow the instructions in the Nymi Connected Worker Platform Administration Guide to delete the user on the Nymi Band. | User deletion is completed successfully on the Nymi Band. | User deletion is completed successfully on the Nymi Band. | Pass |
| 14 | As the End User, follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll a Nymi Band. | Enrollment is completed successfully. | Enrollment is completed successfully. | Pass |
| 15 | Open the MES application. | The application prompts you to login. | Login screen appears. | Pass |
| 16 | Tap the authenticated Nymi Band against a connected NFC reader. | The application log in completes successfully. | Log in succeeds. | Pass |
| 17 | Perform an authentication task in the MES application. | The MES application prompts you for your username and password. | Username and Password prompts appear. | Pass |
| 18 | Tap the authenticated Nymi Band against a connected NFC reader. | The authentication task in the MES application completes successfully. | MES records the authentication event. | Pass |

| System Name: | nes.domain.com |  |  |  |
| --- | --- | --- | --- | --- |
| Test Title | Testing Nymi Band taps to perform e-signatures in a web-based MES from an Android device |  |  |  |
| Purpose | Verify that users can use their Nymi Band to perform e-signatures in web application that they access from an Android device. |  |  |  |
| Test Reference: |  | OQ-015 | Test Run #: | 01 |
| Tester name: |  | Deb Claudio | Execution Date: | 04-17-2025 |

| Test Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| URS tested: URS-020 Functional specification: FS-APP-005 Configuration information: Android application is configured to use the Nymi Connect. |  |  |  |  |
| Step# | Procedure | Expected Result | Actual Result | Pass/Fail |
| 1 | Following the steps provided by the MES vendor to install the MES web application. | Installation was completed successfully. | Installation succeeds. | Pass |
| 2 | Follow the instructions in the Nymi Connected Worker Platform Administration Guide to enroll in a Nymi Band. | Enrollment is completed successfully. | Enrollment succeeds. | Pass |
| 3 | Launch the MES web application. | The web application has opened. | The web application has opened. | Pass |
| 4 | Tap the authenticated Nymi Band against the iOS device. | The e-signature is created successfully. | The e-signature is created successfully. | Pass |